Table of Contents

Preface…………………………………………………………………………..…1

A.  Mission Background………….……………………………………….………4

B.  Atlas Requirements.............................. ..................................................5

C.  The Product….........................................................................................6

D.  Data / Data processing….......................................................................13

E.  Data Structure........…................…….....................................................16

F.  ArcView Project........…....……...............................................................19

- ArcExplorer project....…...................……..............................................20

- Basic Use of ArcView………………………………………………………..20

I.   Advanced Use of ArcView......................................................................21

Appendices…….……………….…….…………………………………………..22

Appendix A. Arc/INFO procedures

Appendix B. Avenue scripts

Appendix C. Directions for Adding 1:24,000 DEM

Appendix D. ArcView Requirements

Preface

This introduction should help orient you before jumping into the accompanying literature and CD.  My name is Jason Seifert and I am a student in the Masters of Geographic Information Science (MGIS) program at the University of Minnesota.  While completing course work during the evening, I have been working full-time at the National Operational Hydrologic Remote Sensing Center (NOHRSC) in Chanhassen, Minnesota.  Formerly named the Snow Survey, NOHRSC is part of the National Weather Service.  My title is GIS/RS Specialist, a position that involves providing GIS support for the office and its customers and performing remote sensing analysis for NOHRSC’s operational products.  My work with NOHRSC was the spark for this, my final research project in the MGIS program.

I work with Dr. Don Cline, a snow hydrologist.  Dr. Cline is coordinating a NASA mission to study the frozen portions of the earth using an experimental satellite.  The mission is the NASA Cold Land Processes Mission, with the designation EX-7 (for experimental satellite #7, I presume).  The paper describing my project mentions the mission and its goals briefly, but the documents contained on the CD deal with that topic more completely.  It is an interesting mission and I  encourage you to look into it further.  I spoke with Dr. Cline about the mission and inquired about the types of geographic data that would be useful at this early stage in the mission.  In short, I was fishing for a subject for my final research project.  We outlined some of the themes that would be useful to scientists involved with the mission and I came up with a plan to create a digital atlas.  This digital atlas serves as a visualization tool and storehouse for data sets used by the scientists.

Preliminary Feedback

Completion of version 1.0 of the digital atlas coincided with a meeting of scientists involved in the Cold Land Processes Mission.  The meeting was a working group to hash out details of a field experiment tentatively planned for 2002.  The working group met in Fort Collins, Colorado on June 27, 2000.  Dr. Cline presented each member of the working group (approximately fifteen people including two NASA representatives) with a copy of the digital atlas.  He then performed a brief demonstration of the digital atlas using a laptop computer and a digital projector.  The digital atlas is an ArcView project consisting of many maps (or "views" in ESRI-speak).  Dr. Cline displayed a number of these maps to demonstrate the types of themes contained in the atlas.  The working group went on a three day road trip, visiting several of potential study areas in northern Colorado.  While in Fraser, Colorado, the working group used the atlas as a decision-making tool.  The group had the task of selecting the preliminary locations of flight lines within the study area.  The locations they select will be used by NASA pilots flying DC-8s equipped with radar during the Field Experiment.  The working group assembled in a small building at the Fraser Experimental Station.  Sofas and chairs were arranged to create an informal theater for viewing the digital atlas, projected on a bare wall.  Using primarily Landsat imagery, a land use/land cover theme, and a roads theme contained in the atlas, the group discussed potential locations for flight lines.  The group reached consensus on the placement of four flight lines.  The group was able to interactively place lines on the landscape while in the field.  The group recognized the value of this type of decision-making.

While I was unable to attend the working group to present the digital atlas, Dr. Cline provided me with feedback he solicited from the group.  Individual reaction to the atlas was positive, with a few minor annoyances.  One nagging problem was the poor performance of the laptop computer and the software in rendering the Landsat image (a 40 Mb JPEG image).  It would routinely take upwards of a minute to draw the image.  This is less of a problem using a stand-alone PC, but a problem that requires attention regardless.  Participants were unable to identify themes they wish had been included in the atlas, but that may change in the future as they use the atlas more and more.  In several instances (during group presentations), someone would request a theme (“what would that look like with rivers?” for example).  In each of these instances, the requested theme was included in the atlas and available for display.

The group of fifteen scientists participating in the working group had varying levels of expertise with ArcView.  Approximately half of the participants indicated that they had access to ArcView software.  Of those respondents, approximately half indicated that they were regular ArcView users.  Dr. Cline estimated that there will be four or five people who will be quite active users of this atlas.  These users and their comments will weigh heavily in decisions made for future versions of the digital atlas.  My task in the short-term is to make the atlas available on the web.  Of primary concern in the organization and distribution of the atlas via the web are atlas versions and data updates.

Participants identified two general uses for the atlas.  The primary utility of the atlas is for general planning.  General planning involves things like flight line planning, creating maps for use in mission reports, and creating derived themes from basic physical layers (soils, LU/LC, DEM, etc.).  Derived themes are themes that will be of use in algorithms, calibration, and snow or hydrologic models.  The working group identified one participant to create some of these derived themes. Future versions of the digital atlas will include derived themes.  The second use of the digital atlas will come during the field experiment.  This is the rather exciting prospect of using a GIS to monitor and update data collection in the field and in real-time or near real-time.  This will require some forethought and expertise to create a mechanism for ingesting field data into the GIS.  The potential benefit of this ingestion will be realized during what has been described as “hot wash” briefings.  This type of briefing typically occurs during the evening after individuals have returned from a day’s worth of field data collection.  The GIS will ingest and process data for display during a briefing.  This will facilitate tracking and visualization of data collected in the field.

Comments and feedback were overwhelmingly positive.  Many participants noted that they were impressed by the level of effort and organization that the atlas represented.  Evidently, many of these experimental missions are more disheveled during early planning stages.  Participants noted that this level of organization would probably stoke the fire of those engaged in preliminary research leading up to the Field Experiment.  They are accustom to seeing this type of product created after the project was over rather than during the planning stages.  The best news of all came from the NASA representatives.  Two days after conclusion of the working group, NASA allocated one half million dollars to fund a preliminary field experiment in March 2001.  While not the sole reason for this allocation, Dr. Cline thought that the professionalism, organization, and effort that the atlas represented weighed considerably in their decision.

## A. Mission Background

The Cold Land Processes Mission is a remote sensing project supported by NASA under the auspices of its Land Surface Hydrology Program (LSHP).  The focus of the mission is the cryosphere, the portion of the earth that is either seasonally or permanently frozen.  Preliminary field and airborne studies are scheduled to begin in July 2001, with the mission to begin in July 2002.  The target launch date for the EX-7 Cold Land Processes Mission satellite is 2005.

The findings of the Cold Land Processes Mission will further the understanding of the cryosphere in terms of hydrology, meteorology, climatology, and ecology.  There are four major science questions for this mission:

- How does the extent of snow and frozen ground affect weather and climate?

- Can the components of the terrestrial cryosphere be observed from space accurately enough to identify meaningful climatic trends?

- To what extent can snow and frozen ground information, deduced from remote sensing data, improve models of cold season processes, hydrologic forecasts and forecasts of high latitude ecosystem functions?

- To what accuracy can snow water equivalent be estimated from remote sensing data, and is this sufficient for hydrological applications

The digital atlas serves as an introductory look at the study area to be examined  during the airborne/field experiments beginning in 2001.    The purpose of the field experiment is to collect data to be used to develop, refine, test, and finalize the microwave remote sensing algorithms that will be used on the experimental satellite.  These data will serve as the “ground truth” in the development of these algorithms.

The study area is located in north central Colorado, encompassing a 3 degree (latitude) by 5 degree (longitude) area.  This area was selected for a number of reasons, with the most distinguishing attribute being the physiographic variety of the landscape contained within a relatively small area.   In addition to the general study area, six intensive study areas (ISAs) have been identified for closer examination.

- North Park

- Rabbit Ears Pass

- Fraser Experimental Forest

- Niwot Ridge

- Grand Mesa

- Upper Gunnison River Basin

## B. Atlas Requirements

The goal of the digital atlas is to create a repository and data visualization tool for data sets relevant to mission researchers.  During the planning phase of the digital atlas project, requirements were established that guided atlas development.

The atlas has to be expandable.  In other  words, the atlas will grow as the mission progresses.  Establishing a logical data structure and tracking metadata are two components of this requirement.  A second requirement is portability.  Much of the research will be performed in the field, so portability allows researchers to bring the atlas into the field.  The digital atlas will help researchers make decisions while at conferences and working groups.  A third requirement is that the atlas should offer familiarization to those not acquainted with the study area.

This project is for fulfillment of the University of Minnesota Masters of Geographic Information Science program.  In a broader sense, this project is designed for use by scientists and administrators of the EX-7 airborne/field survey.  Those who will use the digital atlas may or may not have extensive knowledge of the proposed study area.  The scientists come from various regions of the U.S. and abroad.  This digital atlas serves to orient and familiarize everyone involved in the project.  This will also serve as a framework for further data visualization as the project develops.  As data are collected in the field or the need for more additional data sets is realized, those data will be plugged into this data visualization tool for work in the field and office.

One further requirement of the project is for additional Opportunistic Study Areas (OSAs) to be readily added to the project.  To facilitate this requirement, directions for adding high resolution (1:24,000) DEM to the atlas have been included.  See appendix C..

The audience

The audience has varying levels of GIS familiarity.  Some will use and build upon the existing datasets.  Others will use the free ArcExplorer utility to familiarize themselves with the region.  Still others, though hopefully not many, will use the CD-ROM as a coaster.  A mechanism will be arranged for data to be input into the project framework in real time or near-real time.  Many users of this digital atlas are practicing scientists, so there is an assumption of some familiarity with maps and geographic data.

## C.  The Product

All components of the project are available on the CD. Updates to mission documentation will be updated on the mission website (www.nohrsc.nws.gov/cline/ex7/ex7_home), but the preferred medium for the digital atlas is the CD. The digital medium is flexible, expandable, reproducible, portable, and lends itself to customization.  Users may load the contents of the CD to a local drive to expand upon its contents.

In terms used often in the software industry, this project requires a deliverable.  The deliverable, in this case, is a CD-ROM.  This CD-ROM contains all aspects of the project.  All geographic data are organized within the data directory, all metadata for the respective datasets are organized within the metadata directory, and the text-form explanations of why and how this project was undertaken are organized within the documents directory.  This is all bundled in a user-friendly format that enables users to use this data in a meaningful way.

The fact that a CD-ROM was chosen as the medium for this project fulfills the requirement for portability.  The CD will be loaded onto laptop computers with short notice.  The ability to access the information will be nearly instantaneous.  As the project develops over the coming months, more data will be loaded and new themes will become important.  The version number 1.0 indicates that this is the first in an iterative process.  Adopting the CD-ROM as medium this early in the project should prove valuable in discovering any limitations or opportunities that this medium presents.

ESRI products were chosen as the GIS software of choice for a number of reasons.  ESRI software is the most popular GIS software used today.  Though this product is not  widely distributed (the target audience consists of one to two dozen people), the users are predominantly ESRI GIS users, allowing for ready acceptance from the majority of these users.  I have had limited experience with ArcView and ArcExplorer software platforms.  I have more knowledge of Arc/INFO than with its user-friendlier cousin, ArcView.  In this sense, I viewed this as an opportunity to become more proficient as an ArcView user.  Needless to say, I ran into some difficulty, but believe that the learning curve has flattened out considerably.  The idea for the ArcExplorer project came about when considering the range of potential users of this information.  Most of the users of this data currently have ArcView loaded, but this is not the universal situation.  By using the free, lightweight ArcExplorer data viewer, any PC user is able to utilize this data.

One of the requirements of the digital atlas is for the project to be a scaleable project.  The digital atlas is a first step in a much larger data collection effort.  All data in used in the atlas are in geographic coordinates.  The reasoning for this decision came about as additional themes were added to the project.  This atlas is intended for display and familiarization rather than measurement or navigation purposes where more appropriate projections would become useful.  Some of the prospective data will be remotely sensed images that will be more easily integrated with the atlas.  Field survey crews will transmit data for input into the system using GPS receivers.  Field crew supervisors will be able to track and visualize the data collection effort in real-time.

There are a number of themes depicted in this digital atlas.  These themes may seem quite basic at first glance.  They are quite basic on second and third glance as well.   Having the version number 1.0 indicates that there will be subsequent versions of this atlas with new datasets and more focused Intensive Study Areas.  The scientists participating in this project will identify datasets that require different spatial or temporal resolution.  Prime candidates for improvement are digital terrain and land use/land cover.  The datasets contained within are of reasonable resolution for viewing of the entire region but may prove inadequate when examined at a smaller scale.

The ArcView project contains a time-lapse depiction of precipitation for the study area.  Precipitation data come from PRISM data.  PRISM data are climate layers developed for the coterminous United States. PRISM uses station data and other spatial data sets to estimate patterns of climate in a spatially representative and physically meaningful way.  The station data used in PRISM climate layers comes from National Weather Service Cooperative Observing Network (NWS Coop), the Natural Resources Conservation Service’s SNOTEL network, and local, state, regional and Federal networks.

A script was written in ArcView’s Avenue application language which steps through monthly mean precipitation across the study area.  The user has the ability to step forward or back through the time sequence to view times or areas of specific interest.  See appendix B to reference the Avenue (an ArcView application programming language) scripts developed to accomplish this task.

The following is a complete list of maps in the digital atlas:

- National

The continental US is used as the source of reference for the EX-7 study area defined as 38-41 degrees North and 104-109 degrees West, an area in north central Colorado.

B.  Intensive Study Areas

This view offers the general layout of the study area.  Major highways, cities, and the 6 ISAs are depicted against the outline of the study area.  The roads theme comes from the USGS; roads were extracted from 1:2,000,000 DLG.

C.  Transportation

This offers a slightly more detailed look at the roads of Colorado.  This map is helpful for general navigation across the study area.

D.  Digital Elevation Model

With county boundaries and a 1 degree graticule to use for reference,  this DEM is 3 arc-second DTED resampled to 30 arc-seconds.

E.  Forest Cover Density

Again using county boundaries and a 1 degree graticule, the forest cover density theme comes from the USDA (U.S. Department of agriculture).  Grid cell resolution of this data set is 30 arc-seconds.

F.  Forest Cover Type

Again using county boundaries and a 1 degree graticule, the forest cover type theme comes from the USDA (U.S. Department of agriculture).  Grid cell resolution of this data set is 30 arc-seconds.

G.  Land Use/Land Cover

The Land Use / Land Cover theme is a polygon theme prepared by the EPA and the USGS.

H.  PRISM Precipitation

This theme depicts mean monthly precipitation using precipitation data from Natural Resource Conservation Service (NRCS).  Using the forward and back buttons that appear in the toolbar, the user views a time lapse of mean monthly precipitation, stepping through the calendar month-by-month.  The month and progress are shown in the lower left status bar.

I.  Hospitals

The hospitals theme comes from the Colorado Department of Public Health and the Environment.  Those hospitals designated as level I-IV trauma centers are color coded.  Hospitals in the Denver metropolitan area have been thinned for practical and visual reasons.  Most of the research will be conducted in outlying areas, so the locations of hospitals within the metropolitan area are less significant.

J-O.  Intensive Study Area Maps

1:24,000 DEM are used in these views of the six Intensive Study Areas (ISAs).  River reaches from the National Hydrographic Dataset (NHD) are displayed at this resolution and scale.

ISA Fraser Experimental Forest

ISA Niwot Ridge

ISA Grand Mesa

ISA Rabbit Ears Pass

ISA North Park

ISA Upper Gunnison

- Soils – Dominant Texture Class

From the CONUS-soils database, which is derived from the USDA-NRCS State Soil Geographic Database (STATSGO), this map shows the dominant soil texture class for each of 11 standard layers.  Each grid cell represents 30 arc-seconds.

- Soils – Bulk density

From the CONUS-soils database, which is derived from the USDA-NRCS State Soil Geographic Database (STATSGO), this map shows the dominant soil texture class for each of 11 standard layers.  Each grid cell represents 30 arc-seconds.

S.  Soils – Porosity

From the CONUS-soils database, which is derived from the USDA-NRCS State Soil Geographic Database (STATSGO), this map shows the dominant soil texture class for each of 11 standard layers.  Each grid cell represents 30 arc-seconds.

T.  Soils – Sand, Silt, Clay Fraction

From the CONUS-soils database, which is derived from the USDA-NRCS State Soil Geographic Database (STATSGO), this map shows the dominant soil texture class for each of 11 standard layers.  Each grid cell represents 30 arc-seconds.

V.  Flight Lines

The National Operational Hydrologic Remote Sensing Center (NOHRSC) has established a number of flight lines within the study area.  NOAA aircraft conduct gamma surveys of these flight lines.  Gamma surveys are used to determine the snow water equivalent. These flight lines are shown, as well as airports.  This map is useful not only for seeing the location of currently established flight lines, but will aid in the decision making process regarding the placement of new flight lines for the mission.  There are 41 NOHRSC flight lines within the study area.

W. Snow Information

Against the backdrop of the DEM, various types of snow information collection points are displayed.   The locations of the snow telemetry (snotel), snow pillow, and snow course collection points offer a perspective of the density and placement of the snow information network across the landscape.

X. Landsat

A multiband Landsat TM image is displayed.  The thirty meter resolution offered by the image is a valuable late addition to the atlas.  This resolution is among the highest resolution dataset in the atlas.  Landsat imagery offers perhaps the best depiction of the landscape.  With such high resolution, the Landsat image can be used to assess the strengths and shortcomings of accompanying datasets.

- Hydrography

- Snow Maps 1998-2000

A collection of NOHRSC snow maps is displayed.  Relatively cloud-free maps were selected, on a monthly basis, with maps from the beginning of the month given preference.  Though these maps are low resolution (30 arc-second) since they are created from GOES imagery daily for the entire CONUS, they offer a general idea of the usual extent of the snow pack from January through June.

Themes in the digital atlas:

data/imgfiles/

Landsat image

data/gridfiles/

Terrain

Forest cover density

Forest cover type

Land use / land cover

Soils – texture class

Soils – sand, silt, clay fraction

Soils – bulk density

Soils – porosity

Snowmaps

data/shapfiles/

Roads

Hospitals

Airports

NOHRSC flight lines

Snotel locations

Snow Course locations

Cities

Counties

Basins

Continental divide

National Hydrography Dataset

Hydrography (rivers and lakes)

States

Study area definition

Intensive Study Area definitions

data/co/

Precipitation (monthly and annual)

The intent of this effort was for the product to be simple.  Any user should be able to step through this digital atlas and come away with a good sense of the geography of the region.  The directions for using the CD are available in the README file.  I have opted for presenting the introductory page for the CD in HTML.  The benefit of this is apparent in the ease of navigating to the various documents and images contained on the CD.  HTML  provides a convenient method for accessing the metadata for the project.  While all metadata documents are housed in the same directory, the user is spared the task of navigating through the directory structure.  Instead, the user may simply browse through the documents in now-familiar Internet browser fashion.

## D.   Data / Data processing

I began by collecting spatial data to represent a number of different themes.  Keeping potentially valuable concepts in mind, I searched through NOHRSC data themes.  NOHRSC keeps a number of raster layers on hand  that proved useful.  Two of these themes were forest cover density and forest cover type.  NOHRSC also creates and archives national snow maps.  After realizing the exact dimensions of the study area, I extracted only those portions of the national raster layers that fell within the study area.  Remaining cognizant of projection, it was useful for these themes to be in geographic coordinates.

An interesting task was to collect high resolution DEM for use in the Intensive Study Areas (ISA).  I opted for the highest resolution, free (important consideration) elevation model I could find.  This turned out to be USGS 1:24,000 DEM.  The USGS makes these datasets available in SDTS (spatial data transfer standard) format.  The task became how to import this data format into ArcView.  It turns out that ArcView version 3.2 includes a program module that converts SDTS to Grid format.  Grid is the ASCII grid format utilized in Arc/INFO and ArcView (using the Spatial Analyst extension).  This task is not as easy as simply pressing a button, but a tutorial supplied from the ESRI website (www.esri.com) walked me through the process with little problem.  One hitch I discovered was the inability of the SDTS to Grid conversion tool to deal with level 2 digital elevation data.  The USGS has a grading scale for the 1:24,000 DEM.  The vast majority of these data are graded as level 1, but one of the necessary datasets for this project was graded at level 2.  This may have been an anomaly or may present itself as a problem down the line.  See Appendix C for information regarding the SDTS to grid conversion.

Following the path towards importing the high resolution DEM, the next step was to reproject the raster into geographic coordinates.  The 1:24,000 DEM is kept in UTM projection with NAD27 as their native datum.  While this would be an acceptable projection to use, I elected to reproject the data sets to simplify future use and data assimilation.  I needed the final data layers to be in geographic coordinates using either WGS84 or GRS1980 as the spheroid.  My understanding is that there is an exceptionally small difference between WGS84 and GRS1980.  In most instances the difference is on the order of several meters (generally less) and at the scale of this project would be rendered essentially meaningless.  A limitation of ArcView as a raster processing tool is its inability to reproject raster data.  Realizing this limitation, grid files were brought into Arc/INFO for reprojection.  The conversion from NAD27 to WGS84 is not an appropriate reprojection command.  After considerable hand wringing, I was able to perform the necessary steps to make the conversion.  See appendix A for the specific projection commands necessary to make the conversion.  The operation boils down to a three-step process: 1) reproject from NAD27 to NAD83, 2) redefine the datum to be of the NADCON variety – this a special category created especially for this conversion,  3)  Reproject from NADCON to WGS84.

Many of the themes presented in the digital atlas are extracted from datasets that cover a much larger area than the study area.  Using the geoprocessing extension for ArcView, these datasets were "clipped" to the study area extent.  All datasets containing ex7 in their name have been clipped.  This maintains uniformity throughout the different views and, ideally, keeps the user oriented within this region.

The process for assembling the ArcExplorer project proved to be interesting.  After realizing the utility of an ArcExplorer project, I had set about collecting the various data for the project.  During the data collection process, I came about an ArcExplorer project displaying exactly the type of functionality I wished for.  The ArcExplorer project was bundled with the NRCS PRISM data.  Figuring that imitation is the sincerest form of flattery, I replicated the data structure used in their ArcExplorer project.  PRISM data is organized by state.  Fortunately, the EX-7 study area is contained entirely within the state of Colorado.  I ported the directory structure for the state of Colorado directly into Arc/INFO and proceeded to clip all layers using the study area definition.  I ported them back into my working directory structure.  I proceeded to edit the ArcExplorer project to display various themes not included in the original PRISM ArcExplorer project file.

Data processing of the Landsat image also proved to be interesting.  With no attached metadata, the image was received in TIF format.  No geolocational information, such as a world file, was available.  Projection information was notably absent, but the image appeared to be projected.  After importing the image into Arc/INFO as a grid (IMAGEGRID command), visual inspection led to the conclusion that the image was in Albers conic projection.  Using only the four corners of Colorado as reference, the grid was registered and the projection defined.  The grid was then reprojected to geographic coordinates and exported as a geo-registered TIFF image.  When viewed in Arcview against the National Hydrography Dataset (NHD), the registration of the alignment proved to be good.  The image appeared to be a bit dark initially.  Since there is no image processing available in ArcView, the individual bands were enhanced in a separate piece of software and exported in JPEG format.  This image more closely resembles the enhancement and general look of commonly seen Landsat images.

The soils dataset used for the digital atlas is called CONUS-soils.  CONUS-soils is derived from the USDA-NRCS State Soil Geographic Database (STATSGO).  CONUS-soils is a multi-layer database that uses standardized layers (11 layers: 0-5 cm, 5-10 cm, 10-20 cm, 20-30 cm, 30-40 cm, 40-60 cm, 60-80 cm, 80-100 cm, 100-150 cm, 150-200 cm, and 200-250 cm).  Gridded data was collected for 4 soil themes (bulk density, soil texture class, porosity, and sand, silt, clay fraction).  Additional soil themes are available from the Earth System Science Center at Penn State University (http://www.essc.psu.edu/soil_info/), but were not included in version 1.0.  The Arc/INFO grids were reprojected from the NAD27 datum to the NAD 83 datum then clipped to the extent of the study area

## E. Data Structure

This is a rather simple data structure.  The importance of this simplicity is for a) ready access to data layers by the user and b) ease of adding additional elements to the project.  An important consideration in selecting and maintaining a meaningful data structure is to establish the structure early in the data collection process.  This eliminates extensive file management later in the project.  Establishing a new data structure late in the project greatly complicates the entire project.

avsettings/

data/

gridfiles

imgfiles

shpfiles

DEM

co

documents/

metadata/

The contents of the CD are as follows:

Documents

readme.txt	An introductory text document

ae_tips.txt   	Information on loading and using ArcExplorer software for viewing and printing files

ex7_draft.doc	MS Word file describing the EX-7 Cold Land Processes Mission

Software

aeclient.exe	The executable file that installs ArcExplorer on your PC.  You will need Windows95, or WindowsNT with administrator privileges, to run this program.

Data Directories

1)  co	Directories contain data files grouped according to format (grid, shape, and image).  In addition to these file types, there are a number of Arc/INFO coverages contained in a directory named co.

The following is a list of the data directory for Co, and an explanation of each  of these files.  The co directory contains data used in the ArcExplorer project (Ex7.aep).  Monthly and Annual precipitation data layers from this directory are also used in the ArcView project (Ex7.apr)

dir = data/co

annual  aug   cntyseat   hwy_int   info   jun   may   roads apr   city   dec   hwy_us   jan   lakes   nov   sep counties   feb   hydro   jul   mar    oct   state

Coverage              Description

state                      DCW state boundary

counties                DCW county boundaries

roads                     DCW roads

hwy_int                 Interstate road shields

hwy_us                 US highway road shields

hydro                    DCW hydrology

lakes                     DCW lakes

cntyseat                US Census

annual                   Annual precip polygons w/arc isolines

jan thru dec           Monthly precip polygons

*Note:    DCW refers to coverages directly from the 1:2,000,000 Digital Chart of the World

- gridfiles	This directory contains the Arc/INFO grids used by the ArcView Project.

dir=data/gridfiles

bd  domtext  ex7_fcd  ex7_fct  ex7_lulc  fract  fraser  gld_hill  gunnison  mesa  poros  rbt_ear  rclss1  snomaps  walden

Grid                  Description

bd                     Soils – bulk density

domtext            Soils – texture class

ex7_fcd            Forest cover density

ex7_fct             Forest cover type

ex7_lulc            Land use / land cover

fract                  Soils – sand, silt, clay fraction

fraser                Terrain – 1:24,000 quad in ISA Fraser

gld_hill              Terrain – 1:24,000 quad in ISA Niwot Ridge

gunnison           Terrain – 1:24,000 quad in ISA Upper Gunnison

mesa                 Terrain – 1:24,000 quad in ISA Grand Mesa

poros                 Soils – porosity

rbt_ear              Terrain – 1:24,000 quad in ISA Rabbit Ears Pass

rclss1                 Reclassified 30 arc-second DEM (good for large scale)

snomaps/           Directory containing NOHRSC snowmap grids

walden               Terrain – 1:24,000 quad in ISA North Park

3) shpfiles    This directory contains the ESRI shpaefiles used by the ArcView Project

dir=data/shpfiles

city.shp   ex7_air.shp   ex7_area.shp  ex7_fln.shp  ex7_grat   ex7fct.shp  ex7_bas.shp   ex7_cnty.shp   ex7_div.shp  ex7_hosp2.shp   ex7_hucs.shp  ex7_hydrol.shp  ex7_isa.shp  ex7_lulc.shp  ex7_nhd.shp  ex7_roadtr.shp  ex7_stel.shp   ex7_scrs.shp  states.shp  studarea.shp

Shapefile             Description

city.shp               Cities

ex7_air.shp        Airports

ex7_area.shp     Study area definition

ex7_fln.shp        NOHRSC flight lines

ex7_grat            1-Degree Graticule

ex7_bas.shp      NWS Forecast Basins

ex7_cnty.shp      Counties

ex7_div.shp        Continental divide

ex7_hosp2.shp   Hospitals

ex7_hucs.shp      Hydrologic Units

ex7_hydrol.shp    Hydrography (rivers)

ex7_isa.shp         Intensive Study Area definitions

ex7_nhd.shp       National Hydrography Dataset

ex7_lulc.shp        Land Use / Land Cover

ex7_roadtr.shp    Major Roads

ex7_stel.shp        Snotel locations

ex7_scrs.shp       Snow Course locations

states.shp           States

studarea.shp       Study area definition

4) demfiles:  These DEM files were collected for use in the project.  The DEM files were converted to Arc/INFO files and reprojected for use in the digital atlas. These raw DEM files are not used by the Atlas directly, but are included on the CD.

## F.  The ArcView Project (Ex7.apr)

The contents of this project are contained in two formats.  The primary and more comprehensive of the two is an ArcView project (EX7.apr).  This format is for use with ESRI’s ArcView software.  The second is ESRI’s ArcExplorer project (EX7.aep).   This format is for use with ESRI’s free geodata viewer, ArcExplorer.

The Arcview project is located at the top of the CD’s directory structure so that it is one of the first things the user sees when viewing the contents of the CD-ROM.  The most significant feature of this ArcView project is that many controls are not visible so that the user has only limited functionality.  This was done to simplify navigation by digital atlas users.   Users who would like the full-fledge functionality of the software, would like to manipulate the data, or add additional data layers should refer to section H. Advanced use of ArcView.

## G.  The ArcExplorer Project (Ex7.aep)

The use of ArcExplorer is recommended for users who do not have access to ArcView and the Spatial Analyst extension.  The aeclient.exe program should be saved to a local drive.  This program will guide the user through ArcExplorer installation.  After installation, double-clicking on the Ex7.aep file will begin the ArcExplorer session.  The ArcExplorer software does not have the same capabilities as ArcView software, so it should be noted that many datasets (gridded) cannot be viewed using the ArcExplorer project.  This software platform has limited display capabilities that can make for awkward map appearance.

## H. Basic use of ArcView

Opening the ArcView project (ex7.apr) will kick off your ArcView session.  In essence, this project is a collection of “Views”, with each View consisting of an arrangement of map themes.  Double-clicking on a map name makes that view active.  Map theme attributes have been set, but may be altered by double-clicking on the themes name in the map legend.  Additionally, the check box next to each theme in the legend indicates whether that theme is visible.

The order of display of each map theme is determined by its position in the legend.  To change the order of display (making highways display on TOP of rivers for example), users need only drag and drop the map theme to a different position within the map legend.

## I.  Advanced use of ArcView

Users may elect to copy the project file to a local drive so that they might make alterations to the project.  This is a necessity since the project file is read-only on the CD-ROM.  Additionally, users may also want to copy the entire project to a local drive if they plan on generating additional themes (though not  a necessity, this dramatically simplifies file management).

To make the ArcView project portable, all pathnames referenced in the project file (.apr) are relative.  By default, ArcView saves absolute pathnames.  This was remedied by editing the project file using a text editor.  If a user copies the project file to their local drive, they should edit the project file so that the pathnames point to the CD drive or, if the data was also copied to the local drive, the pathname should point to the new location of the data/ directory.  For example: suppose the CD drive on your PC is named f:

The user should edit the project file, performing a find and replace action which looks something like this:

Find: data/

Replace with: f:/data/

If the user copies the data/ directory and the project file to the same location, they do not need to edit the project file since the relative pathnames will still point to the correct location.

The look of some of the data themes has been recorded as ArcView legend files (*.avl).  These come in handy if you elect to create new maps and would like to retain consistency with previously created maps.  After adding a theme to a new view, it is often the case that the look (line types, colors, classification schemes, etc.) needs to be altered using the edit theme legend dialog.  When using this dialog, the user may use the “Load…” option to assign predefined settings for some of the primary themes in the atlas.  Legend files for primary themes have been saved in the Avsettings/ directory.

## Appendix A.  Arc/INFO Procedures

Arc/INFO was used primarily as a tool for reprojecting grids and coverages. These are example Arc/INFO commands for reprojecting USGS 1:24,000 DEM in UTM projection to Geographic coordinates using the WGS 84 spheroid.  The naming convention in this example begins with the original grid “grdname” with the final grid being “grdgeo.”  See also the Arc/INFO help documentation for “NAD to WGS”

1) Reproject from NAD27 to NAD83

Grid:  grdcon = project (grdname)

Project:  output

Project:  projection utm

Project:  zone 13

Project:  datum NAD83

Project:  units meters

Project:  parameters

Project:  end

2) Redefine the datum to be of the NADCON variety – this a special category created especially for this conversion.

Arc:  projectdefine grid grdcon

Project: datum NAR_C

Project: parameters

3) Reproject form NADCON to WGS84

Grid: grdgeo = project (grdcon)

Project: output

Project: projection geographic

Project: units dd

Project: datum WGS84

Project: parameters

Project: end

## Appendix B. Avenue Scripts

Avenue is an application programming language used with ESRI’s ArcView software.  Avenue scripts were written to add some functionality to the PRISM Precipitation map.  Three buttons (back, cycle, and forward) are activated on the toolbar when the PRISM Precipitation view is active.   The prism_view avenue script is used for creating a time lapse of mean monthly precipitation.  The button with the cycle icon is used to activate this script.  The lapse rate may be set by altering a variable within this script.  A second script named wait.sec is called from this program.

theView = av.GetActiveDoc

theDisplay = av.GetActiveDoc.GetDisplay

if ( _thmList.IsEmpty ) then

_thmList = {}

_thmList.Add(theView.FindTheme("January"))

_thmList.Add(theView.FindTheme("February"))

_thmList.Add(theView.FindTheme("March"))

_thmList.Add(theView.FindTheme("April"))

_thmList.Add(theView.FindTheme("May"))

_thmList.Add(theView.FindTheme("June"))

_thmList.Add(theView.FindTheme("July"))

_thmList.Add(theView.FindTheme("August"))

_thmList.Add(theView.FindTheme("September"))

_thmList.Add(theView.FindTheme("October"))

_thmList.Add(theView.FindTheme("November"))

_thmList.Add(theView.FindTheme("December"))

end

av.ShowStopButton                ' Displays a stop button

i = 0

for each thm in _thmList

theCount = _thmList.Count

i = i + 1

progress = ( i/theCount) * 100

doMore = av.SetStatus(progress)

if ( doMore.Not) then

break

end

av.ShowMsg(thm.AsString)

thm.SetVisible(true)

theDisplay.flush

' Change this variable to increase or decrease the lapse rate

av.Run("wait.sec", {1})

theDisplay.flush

thm.SetVisible(false)

end

The wait.sec script is called by the prism_view script.  A timing function is employed to delay the display.

'Called  the prism_view  script using av.run("wait.sec", {X} )

'where X is the number of seconds you want to delay

theView = av.GetActiveDoc

numWaitSec = Self.Get(0)

startTime = Date.Now.AsSeconds

stopTime = startTime + numWaitSec

theTime = Date.Now.AsSeconds

while (theTime < stopTime)

theTime = Date.Now.AsSeconds

end

The prism_step_forward avenue script is also employed for viewing PRISM precipitation data. It steps forward one month at a time.  The forward button activates this script.  A similar script named prism_step_back is activated with the back button.

theView = av.GetActiveDoc

theDisplay = theView.GetDisplay

if ( _thmList.IsEmpty ) then

_thmList = {}

_thmList.Add(theView.FindTheme("January"))

_thmList.Add(theView.FindTheme("February"))

_thmList.Add(theView.FindTheme("March"))

_thmList.Add(theView.FindTheme("April"))

_thmList.Add(theView.FindTheme("May"))

_thmList.Add(theView.FindTheme("June"))

_thmList.Add(theView.FindTheme("July"))

_thmList.Add(theView.FindTheme("August"))

_thmList.Add(theView.FindTheme("September"))

_thmList.Add(theView.FindTheme("October"))

_thmList.Add(theView.FindTheme("November"))

_thmList.Add(theView.FindTheme("December"))

end

theVisibleThemes = theView.GetVisibleThemes

foundMonth = FALSE

for each thm in theVisibleThemes

theIndex = _thmList.Find(thm)

if ( ( theIndex <> -1 ) and ( theIndex < _thmList.Count) ) then

_thmList.Get(theIndex).SetVisible(False)

if (theIndex <> 11) then

_thmList.Get(theIndex+1).SetVisible(True)

foundMonth = TRUE

end

theDisplay.Flush

break

end

end

if ( foundMonth.Not ) then

_thmList.Get(0).SetVisible(True)

theDisplay.Flush

end

## Appendix C.  Directions for adding 1:24,000 DEM to the Atlas.

The first step in the process is to determine which 1:24,000 topo quads are needed.  The USGS website (www.usgs.gov) has a number of methods for extracting quad names based on location.  Once the areas have been identified, collecting the data and importing the datasets into ArcView can be accomplished using the directions supplied in a tutorial on the ESRI website.  The tutorial is available at http://www.esri.com/news/arcuser/0100/webdata8.html.  The data is available from the USGS at http://edcwww.cr.usgs.gov/doc/edchome/ndcdb/ndcdb.html.

## Appendix D.  ArcView Requirements

It is necessary to have ArcView software as well as the Spatial Analyst Extension for ArcView.  ArcView software runs on:

Windows

Windows 95/98

Windows NT 4.0

Windows 2000

UNIX

Compaq Tru64 UNIX

HP 9000/700 and 8x7

IBM RS/6000

SGI

Sun Solaris 2.x